an AMP company

# Component Product Specification

| # MAL Business Unit # Humphrys Road # Dunstable, Beds # LU5 4SX, UK # Tel: + 44 (0) 1582 471200 # Fax: + 44 (0) 1582 472277 |  | # Description: # Part Number: # Application: | # Surface Mount VCO # MLO80100-01370 # RF,Base Station | # Spec. No. # Issue: # Date: | # DV2/0199/01 # A # 11/01/1999 |
| --- | --- | --- | --- | --- | --- |

| 1. ELECTRICAL :- Applies over output frequency range @ +25°C, VCC +5V, output load 50  , unless otherwise stated. Limits & Conditions are indicated values |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter |  |  | Min. | Typ. | Max. |  | Units |  | Conditions |  |  |  |
| Frequency Range (FOUT) |  |  | 1340 |  | 1400 |  | MHz |  | Over TOP |  |  |  |
| Tuning Voltage (VT) |  |  | +0.5 |  | +4.5 |  | V |  | Over TOP |  |  |  |
| RF Output Power (POUT) |  |  | -2.0 |  | +4.0 |  | dBm |  | Over TOP |  |  |  |
| Phase Noise |  |  |  | -122 | -118 |  | dBc/Hz |  | SSB at 100 kHz offset from carrier |  |  |  |
| Tuning Sensitivity |  |  |  | 30 |  |  | MHz/V |  |  |  |  |  |
| Tuning Linearity |  |  |  | 1.2 | 2.0 |  | Ratio |  |  |  |  |  |
| Harmonic Output |  |  |  | -13 | -10 |  | dBc |  |  |  |  |  |
| Frequency Pushing |  |  |  | 3.0 | 4.0 |  | MHz/V |  | VCC +5V  0.25V |  |  |  |
| Frequency Pulling |  |  |  | 2.0 | 5.0 |  | MHz |  | 1.5:1 VSWR load, all phases |  |  |  |
| Frequency Drift |  |  |  | -0.12 |  |  | MHz/C |  | Over TOP |  |  |  |
| Tune Input Capacitance |  |  |  |  | 50 |  | pF |  |  |  |  |  |
| Supply Current (ICC) |  |  |  |  | 15 |  | mA |  |  |  |  |  |
| Supply Voltage (VCC) |  |  | +4.75 |  | +5.25 |  | V |  | VCO Oscillates |  |  |  |
| 2. MECHANICAL |  |  |  |  |  |  |  |  |  |  |  |  |
| Package Outline : 1107962-4 Type Of Package : LSM1 Leadless Surface Mount |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. ENVIRONMENTAL |  |  |  |  |  |  |  |  |  |  |  |  |
| - 40 to +85 C case operating temperature (TOP) - 45 to +120 C storage temperature (TSTOR) |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. AMENDMENTS, specifications subject to change without notice |  |  |  |  |  |  |  |  |  |  |  |  |
| Date | Initials | Details |  |  |  |  |  |  |  |  |  | Issue |
| Date |  |  |  |  |  |  |  |  |  |  |  |  |
| DO Issue |  |  |  |  |  |  |  |  |  |  |  |  |
| Change Note |  |  |  |  |  |  |  |  |  |  |  |  |
| Approved | Q.A. | Title: CPS for MLO80100-01370 |  |  |  |  |  | Number: DV2/0199/01 |  |  | Sheet 1 of 1 |  |